УТВЕРЖДАЮ: Начальник управления сигнализации и связи АО «Узбекистон темир йуллари» _______________ Акботаев Т.А. «____»________________2026 г.

ТЕХНИЧЕСКОЕ ЗАДАНИЕ

Камера видеонаблюдения.

Согласовано:

Заместитель начальника

управления сигнализации и связи                                                Шерматов Т.С.

Ташкент 2026 г.

РАЗДЕЛ 1. ОБЩИЕ СВЕДЕНИЯ

Подраздел 1.1 Наименование

Камера видеонаблюдения

Подраздел 1.2 Основание и цель приобретения товара

Подраздел 1.3 Сведения о новизне (год производства/выпуска товара)

Товар должен быть новым и не ранее 2026 года производства

Подраздел 1.4 Этапы разработки / изготовления

Соответствие техническим нормативным документациям завода изготовителя

Подраздел 1.5 Документы для разработки / изготовления

Соответствий межгосударственными стандартам или аналогичный общепринятый стандарт, не уступающий или превосходящий по техническим и функциональным параметрам

Подраздел 1.6 Код ТН ВЭД и другие международные коды при применимости

Будет определяться после заключения договора на поставку

РАЗДЕЛ 2. ОБЛАСТЬ ПРИМЕНЕНИЯ

Установка камеры видеонаблюдения на опорах

3. УСЛОВИЯ ЭКСПЛУАТАЦИИ

Подраздел 3.1 Общие условия эксплуатации

Должен быть рассчитан на работу в условиях умеренного климата. Рабочий диапазон температур – 30°C ~ 60°C, Рабочая влажность– 5%≤ 95 % , Класс защиты IP66; Оборудование АнтиКоррозия Класс коррозионной стойкости C Рейтинг IK IK10 (без смотрового окна и воздуходувки) защита от перенапряжения,

Подраздел 3.2 Дополнительные/специальные требования к эксплуатации

Внутри коробки необходима быть крепления, блок питание.

Подраздел 3.3 Требования к расходам на эксплуатацию товара

Производитель/поставщик предоставляет информации о сроке службе камеры видеонаблюдение и эксплуатационным расходам.

РАЗДЕЛ 4. ТЕХНИЧЕСКИЕ ТРЕБОВАНИЯ

Подраздел 4.1 Технические, функциональные и качественные характеристики (потребительские свойства) товаров Main control Computing power 1 TOPS Memory DDR3 1.0 GB Flash type SPI NAND FLASH 256.0 MB Camera Focal length 5.3-175mm Max Aperture F1.6(W)-F4.4(T) Image sensor 1/2.7" CMOS/5MP 2880(H)*1620(V) Lens aperture type DC iris+Hall sensor Angle of view Horizontal AoV: 2.4° (T) to 54° (W) Vertical AoV: 1.7°(T) to40.2°(W) AoV deviation: ≤ 5% IR range 150m Illumination distance 30m Zoom Digital zoom: 16x; Optical zoom: 33x IR Wavelength 850 nm DORI D(m) O(m) R(m) I(m) W 106.0 42.7 21.2 10.6T 3500.0 1411.3 700.0 350.0PTZ PTZ rotation angle Pan: 0°～360° Vertical-15°～90° Pan speed Maximum speed for rotating to a preset position: 100°/s; manual control speed: 0.1°/s to 100°/s Note: The PTZ rotation speedneeds to be limited in the case of low temperature. Temperaturefor entering the speed limit mode: –20°C Temperature for exitingthe speed limit mode: –10°C Max pan speed: 30°/s Max tilt speed: 20°/s Tilt speed Maximum speed for rotating to a preset position: 80°/s; manual control speed: 0.1° /s to 80°/s Note: The PTZ rotation speed needs to be limitedinthecase of low temperature. Temperature for entering the speedlimit mode: –20°C Temperature for exiting the speed limit mode: –10°CMax pan speed: 30°/s Max tilt speed: 20°/s Park action Supported Tour Supported Preset position 512 (including one home position) Home position One home position Target/Body Target Target detection/Target attribute recognition Person Person detection Fast movement/Tripwire crossing/Intrusion/Area entry or exit Technical Specifications Other intelligences Behavior analysis /Loitering Crowd flow analysis Queue length/Head counting/Crowd flow statistics/AttendanceIntelligent tracking Supported Defocus Supported Basic Event Motion detection/Abandoned object detection/Removedobject detection Alarm information HTTPS certificate expiration/SDK certificate expiration/Appverification certificate expiration/Level-2 app verification certificateexpiration/Mobile app certificate expiration/SDK root certificateexpiration/SDK revocation certificate expiration/GB 35114 certificateexpiration/802.1X certificate expiration/Mobile app root certificateexpiration/Log certificate expiration/Network disconnection/Networkpacket receiving anomaly/Full hard disk/Hard disk removal/Failuretowrite data to disks/Media stream interrupted/Log savinganomaly/Flash read/write/Temperature sensor fault/Hot swappingof network adapters/Video loss/Lens PT optocoupler fault/PTZconfiguration anomaly/PTZ optocoupler fault/Position changeOpen ecosystem Algorithm openness Open OS Supported Data openness Data openness SDK (proprietary protocol)/REST API (proprietary protocol)/GA/T1400/GB/T 28181 (2011, 2016) /ONVIF Profile S/G/T/M Target/Body Target Target detection/Target attribute recognition Person Person detection Fast movement/Tripwire crossing/Intrusion/Area entry or exit Technical Specifications Other intelligences Behavior analysis /Loitering Crowd flow analysis Queue length/Head counting/Crowd flow statistics/AttendanceIntelligent tracking Supported Defocus Supported Basic Event Motion detection/Abandoned object detection/Removedobject detection Alarm information HTTPS certificate expiration/SDK certificate expiration/Appverification certificate expiration/Level-2 app verification certificateexpiration/Mobile app certificate expiration/SDK root certificateexpiration/SDK revocation certificate expiration/GB 35114 certificateexpiration/802.1X certificate expiration/Mobile app root certificateexpiration/Log certificate expiration/Network disconnection/Networkpacket receiving anomaly/Full hard disk/Hard disk removal/Failuretowrite data to disks/Media stream interrupted/Log savinganomaly/Flash read/write/Temperature sensor fault/Hot swappingof network adapters/Video loss/Lens PT optocoupler fault/PTZconfiguration anomaly/PTZ optocoupler fault/Position changeOpen ecosystem Algorithm openness Open OS Supported Data openness Data openness SDK (proprietary protocol)/REST API (proprietary protocol)/GA/T1400/GB/T 28181 (2011, 2016) /ONVIF Profile S/G/T/M Network transmission Protocols supported by IPv4 TCP, UDP, IPv4, DHCP, DNS, ICMP, IGMP, HTTPS, FTP, SFTP, RTP, RTSP, RTCP, SIP, ARP, SSL, NTP, SNMP (V1/V2/V3), 802.1X, QoS, SMTP, and PPPoE. Note: The northbound protocol needs toadapt to the PPPoE dialup scenario. Protocols supported by IPv6 HTTPS,RTP,RTCP,RTSP,Onvif,IPv6,ICMPv6,DHCPv6 Web Browser Web Browser 32- or 64-bit Windows 7, Windows 8, or Windows 10 GoogleChrome: 32-bit Google Chrome 38–49 (The IE Tab plug-inmust beinstalled for Google Chrome 46–49.) 32- or 64-bit Google Chrome50+ Internet Explorer: 32-bit Internet Explorer 10 and 11 Firefox: 32-bit Firefox 35–51 32- or 64-bit Firefox 70+ Microsoft Edge: 32- or 64-bit Microsoft Edge 50+ Restrictions: Compatibility problems mayoccur on the Firefox browser. (Pages may be out of order for ashorttime.) Security Data security Media security Supports web- or SDK-based media stream encryption; supportsamaximum of two channels of encrypted media streams; mediastream encryption and full intelligence, target recognition, or imageupload through SFTP/FTP cannot be enabled at the same time; afterthe intelligent analysis function is enabled, only one-channel streamencryption is supported; supports encrypted storage of datainSDcards. By default, RTSP uses MD5 digest authentication. Video digital watermark Supported Image digital watermark Supported Service security User rights Three user roles: administrator, advanced operator, and commonoperator Security mode User name and password authentication, 802.1X authentication, andHTTPS digital certificate Secure boot Supported System intrusion detection Promiscuous network adapter detection;Sensitive systemfiledetection;Unauthorized super account detection;Botnet detection; Rootkit detection;Trustlisted program detection ;Privilege escalationdetection Port Storage port One microSD card slot that supports microSDHC/microSDXC (maximummemory cardcapacity: 256 GB) Alarm port 2 input port and 1 output port Network port One RJ45 10M/100Madaptive Ethernet port Audio port Supported (one-channel LINE IN and one-channel built-in microphone)/Supported (one- channel LINE OUT or one-channel speaker) General Specifications Power type AC24V,POE(IEEE 802.3At) Power consumption Maximum:22.3W,typical:8.9W Operating temperature –30°C to +60°C Storage temperature -40℃ to 70℃ Operating humidity 5% to 95%(non-condensing) Surge protection 6kV IP rating IP66 Equipment Anti Corrosion Class C Weight Net weight 2.6kg;Gross weight 4.5kg Dimensions Device dimensions 202.6mm*202.6mm*260mm;Package dimensions 285mm*285mm*375mmLanguage Simplified Chinese, English, French, Spanish, Portuguese (Europe), Portuguese (Brazil), Thai, Italian, and Turkish

Подраздел 4.2 Требования к маркировке

Согласно нормативно-технической документации завода изготовителя.

Подраздел 4.3 Требования к размерам и упаковке

Согласно нормативно-технической документации завода изготовителя.

РАЗДЕЛ 5. ТРЕБОВАНИЯ ПО ПРАВИЛАМ СДАЧИ И ПРИЕМКИ

Подраздел 5.1 Порядок сдачи и приемки

Согласно нормативно-технической документации завода изготовителя. Приемка оборудование осуществляется на территории Покупателя.

Подраздел 5.2 Требования по передаче заказчику технических и иных документов при поставке товаров

Сертификат качества, сертификат соответствие, технический паспорт, сертификат происхождения товара.

Подраздел 5.3 Требования к страхованию товара

Страхование осуществляется в соответствии с условиями договора поставки, а также договора на оказание транспортных услуг между заводом изготовителем и транспортной компанией

РАЗДЕЛ 6. ТРЕБОВАН ИЯ К ТРАНСПОРТИРОВАНИЮ

Согласно нормативно-технической документации завода изготовителя.

РАЗДЕЛ 7. ТРЕБОВАНИЯ К ХРАНЕНИЮ

Хранение материала допускается в упакованном виде в закрытых помещениях. В условиях, соответствующих группе по ГОСТу 15150-69 срок хранение один год.

РАЗДЕЛ 8. ТРЕБОВАНИЯ К ОБЪЕМУ И/ИЛИ СРОКУ ПРЕДОСТАВЛЕНИЯ ГАРАНТИЙ

Необходимо представить гарантию на соответствие технических характеристик закупаемого товара камеры видеонаблюдение. Гарантийный срок поставляемого материала должен составлять 1 год со дня ввода в эксплуатации при условии предварительного хранения не более 6 месяцев со дня изготовления.

РАЗДЕЛ 9. ТРЕБОВАНИЯ К ОБСЛУЖИВАНИЮ

Обслуживание и эксплуатация осуществляется согласно, паспорту и руководство по эксплуатации производителя.

РАЗДЕЛ 10. ЭКОЛОГИЧЕСКИЕ ТРЕБОВАНИЯ

В соответствии с нормами и правилами, действующими в Республики Узбекистан

РАЗДЕЛ 11. ТРЕБОВАНИЯ ПО БЕЗОПАСНОСТИ

Материал камеры видеонаблюдение должен соответствовать международным стандартам.

РАЗДЕЛ 12. ТРЕБОВАНИЯ К КАЧЕСТВУ

Согласно нормативно-технической документации завода изготовителя.

РАЗДЕЛ 13. ДОПОЛНИТЕЛЬНЫЕ (ИНЫЕ) ТРЕБОВАНИЯ

Требования к материалу камеры видеонаблюдение - заказываемый материал должен быть укомплектован полностью: Требования к шефмонтажу и другим работам – не требуется; Требования к послегарантийному обслуживанию - не требуется; Требования к обучению персонала – не требуется;

РАЗДЕЛ 14. ТРЕБОВАНИЯ К КОЛИЧЕСТВУ, КОМПЛЕКТАЦИИ, МЕСТУ И СРОКУ (ПЕРИОДИЧНОСТИ) ПОСТАВКИ

Поставка товара 100% на основании заявок , поставка товара в течении 10-дней с момента заключения контракта, - поставка железнодорожным транспортом на условиях DAR АО «Темирйўлинфратузилма» - отгрузка автомобильным транспортом Республики Узбекистан, г. Ташкента, Тарас Шевченко 7дом. -авиа транспортом Республика Узбекистан, г. Ташкент, «Международный аэропорт Ташкент» имени Ислама Каримова

РАЗДЕЛ 15. ТРЕБОВАНИЕ К ФОРМЕ ПРЕДСТАВЛЯЕМОЙ ИНФОРМАЦИИ

Особые требования к форме представляемой информации не требуются.

РАЗДЕЛ 16. ПЕРЕЧЕНЬ ПРИНЯТЫХ СОКРАЩЕНИЙ

| № п/п | Сокращение | Расшифровка сокращения |
| --- | --- | --- |
| 1 | Ш | Управление сигнализации и связи |
| 2 | PTZ | камера, которая поддерживает удалённое управление направлением и увеличением |

Разработал:

Начальник отдела видеонаблюдения

управления сигнализации и связи                                                             Кайимов Ғ.А.